STING TAGS v9.6  ·  v4  —  Label Configuration & Warning Parameters Guide

# 6. Category Label Tables — All 45 Categories

Each category shows the complete Edit Label table (purple header) followed immediately by its warning configuration (amber header). Follow rows top to bottom without skipping.

Columns: # = sequence | Tier = T1/T2/T3 | Parameter = exact parameter name | Prefix/Suffix = text to type in those fields | Spc = space (usually 0) | Calculated Value Formula = exact text to type in Calculated Value dialog.

## Quick Formula Reference

| Tier | Name field (must be unique per row) | Discipline | Type | Calculated Value Formula |
| --- | --- | --- | --- | --- |
| Tier 1 | — | — | — | No formula — add parameter directly (no Calculated Value) |
| Tier 2 | Show Tier 2 - <row#> | Common | Text | if(TAG_PARA_STATE_2_BOOL, <PARAM>, "") |
| Tier 3 | Show Tier 3 - <row#> | Common | Text | if(TAG_PARA_STATE_3_BOOL, <PARAM>, "") |
| Warning | if(TAG_WARN_VISIBLE_BOOL, <WARN_PARAM>, "") |

## Change Log — v4 (Parameter Name Corrections)

Cross-referenced all 161 guide parameters against MR_PARAMETERS.TXT. 14 parameters were renamed to match actual shared parameter file names. 15 new parameters were identified as missing and must be appended to MR_PARAMETERS.TXT.

| # | Old Name in Guide | Correct Name in MR_PARAMETERS.TXT | Used In |
| --- | --- | --- | --- |
| 1 | ELC_PNL_SHORT_CIRCUIT_KA | ELC_PNL_SHORT_CIRCUIT_RATING_KA | Electrical Equipment |
| 2 | ELC_PNL_SPARE_WAYS_NR | ELC_SPARE_WAYS_NR | Electrical Equipment |
| 3 | HVC_BMS_CONTROL_TYPE_TXT | HVC_CONTROL_TYPE_TXT | Mechanical Equipment |
| 4 | ELC_PNL_IP_RATING_TXT | ELC_IP_RATING_TXT | Electrical Equipment |
| 5 | HVC_INSULATION_TXT | SYS_INSULATION_TYPE_TXT | Ducts |
| 6 | PLM_INSULATION_TXT | SYS_INSULATION_TYPE_TXT | Pipes |
| 7 | BLE_CEIL_GRID_MM | BLE_CEILING_GRID_SZ_MM | Ceilings |
| 8 | BLE_DOOR_OPERATION_TYPE_TXT | BLE_DOOR_OPERATION_TYPE_SWING_SLIDE_FOLD_ROLL_TXT | Doors |
| 9 | BLE_FLR_TYPE_TXT | BLE_FLR_TYPE_CLASSIFICATION_TXT | Floors |
| 10 | BLE_ROOF_PRIMARY_MAT_TXT | BLE_ROOF_TYPE_TXT | Roofs |
| 11 | ELC_CBL_DEST_TXT | ELC_CBL_DESTINATION_TO_TXT | Conduits |
| 12 | ELC_CBL_SOURCE_TXT | ELC_CBL_SOURCE_FROM_TXT | Conduits |
| 13 | HVC_DCT_TERMINAL_SOUND_DB_NR | HVC_DCT_SOUNDLVL_DB | Air Terminals |
| 14 | AREA_SQ_M | ASS_ROOM_AREA_SQ_M | Rooms |

New Parameters — Must Be Added to MR_PARAMETERS.TXT

| # | Parameter Name | Type | Group | Used In |
| --- | --- | --- | --- | --- |
| 1 | BLE_DOOR_FRAME_MAT_TXT | TEXT | BLE_ELES | Doors |
| 2 | BLE_DOOR_GLAZING_TYPE_TXT | TEXT | BLE_ELES | Doors |
| 3 | ELC_CBL_FEEDER_SZ_MM2 | NUMBER | ELC_PWR | Electrical Equipment |
| 4 | WARN_BLE_RAMP_SLOPE_PCT_RAMPS | TEXT | RGL_CMPL | Ramps |
| 5 | WARN_ELC_VLT_DROP_PCT_ELECTRICAL_EQUI | TEXT | ELC_PWR | Electrical Equipment, Electrical Fixtures |
| 6 | WARN_FLS_SFTY_COVERAGE_AREA_SQ_M_SPRINKLERS__FIR | TEXT | FLS_LIFE_SFTY | Sprinklers, Fire Alarm Devices |
| 7 | WARN_HVC_DCT_SOUNDLVL_DB | TEXT | HVC_SYSTEMS | Air Terminals |
| 8 | WARN_HVC_EFF_RATIO_NR_MECHANICAL_EQUI | TEXT | HVC_SYSTEMS | Mechanical Equipment |
| 9 | WARN_HVC_VEL_MPS_FLEX_DUCTS | TEXT | HVC_SYSTEMS | Flex Ducts |
| 10 | WARN_PER_SUST_CARBON_FOOTPRINT_KG_WALLS__FLOORS__ | TEXT | PER_SUST | Walls, Floors, Roofs |
| 11 | WARN_PER_THERM_U_VALUE_W_M2K_NR_FLOORS | TEXT | PER_SUST | Floors |
| 12 | WARN_PER_THERM_U_VALUE_W_M2K_NR_ROOFS | TEXT | PER_SUST | Roofs |
| 13 | WARN_PER_THERM_U_VALUE_W_M2K_NR_WALLS | TEXT | PER_SUST | Walls |
| 14 | WARN_PLM_PPE_FLW_LPS_PIPES__HOT_WATE | TEXT | PLM_DRN | Pipes |
| 15 | WARN_RGL_ACCESS_CLEAR_WIDTH_MM_DOORS__RAMPS__C | TEXT | RGL_CMPL | Doors, Ramps |

## 6A — Architecture

Family: STING - Wall Tag    TAG7 Container: ARCH_TAG_7_PARA_WALL_TXT

| Walls |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T1 | BLE_WALL_FUNCTION_TXT |  |  | 0 |  | — | — | — | Add directly |
| 3 | T1 | BLE_WALL_THICKNESS_MM | [ | mm] | 1 | ✓ | — | — | — | Add directly |
| 4 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 5 | T2 | BLE_WALL_FUNCTION_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, BLE_WALL_FUNCTION_TXT, "") |
| 6 | T2 | BLE_WALL_TYPE_CLASSIFICATION_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, BLE_WALL_TYPE_CLASSIFICATION_TXT, "") |
| 7 | T2 | BLE_WALL_PRIMARY_WALL_MAT_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, BLE_WALL_PRIMARY_WALL_MAT_TXT, "") |
| 8 | T2 | BLE_WALL_CST_METHOD_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, BLE_WALL_CST_METHOD_TXT, "") |
| 9 | T2 | PER_THERM_U_VALUE_W_M2K_NR |  |  | 0 |  | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, PER_THERM_U_VALUE_W_M2K_NR, "") |
| 10 | T2 | PER_THERM_R_VALUE_M2K_W | ¦ R= |  | 0 |  | Common | Text | Show Tier 2 - 10 | if(TAG_PARA_STATE_2_BOOL, PER_THERM_R_VALUE_M2K_W, "") |
| 11 | T2 | PER_THERM_CONDENSATION_RISK_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 11 | if(TAG_PARA_STATE_2_BOOL, PER_THERM_CONDENSATION_RISK_TXT, "") |
| 12 | T2 | FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN | FRR: |  | 0 |  | Common | Text | Show Tier 2 - 12 | if(TAG_PARA_STATE_2_BOOL, FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN, "") |
| 13 | T2 | BLE_WALL_SOUND_TRANSMISSION_CLASS_RATING_NR | ¦ STC: | dB | 0 | ✓ | Common | Text | Show Tier 2 - 13 | if(TAG_PARA_STATE_2_BOOL, BLE_WALL_SOUND_TRANSMISSION_CLASS_RATING_NR, "") |
| 14 | T2 | BLE_WALL_INTERIOR_FINISH_TXT | Int: |  | 0 |  | Common | Text | Show Tier 2 - 14 | if(TAG_PARA_STATE_2_BOOL, BLE_WALL_INTERIOR_FINISH_TXT, "") |
| 15 | T2 | BLE_WALL_EXTERIOR_FINISH_TXT | ¦ Ext: |  | 0 |  | Common | Text | Show Tier 2 - 15 | if(TAG_PARA_STATE_2_BOOL, BLE_WALL_EXTERIOR_FINISH_TXT, "") |
| 16 | T2 | BLE_WALL_MOISTURE_BARRIER_INCLUDED_TXT | ¦ Moisture: |  | 0 | ✓ | Common | Text | Show Tier 2 - 16 | if(TAG_PARA_STATE_2_BOOL, BLE_WALL_MOISTURE_BARRIER_INCLUDED_TXT, "") |
| 17 | T2 | PER_SUST_RECYCLED_CONTENT_PCT | Recycled: | % | 0 |  | Common | Text | Show Tier 2 - 17 | if(TAG_PARA_STATE_2_BOOL, PER_SUST_RECYCLED_CONTENT_PCT, "") |
| 18 | T2 | PER_SUST_CARBON_FOOTPRINT_KG | ¦ CO₂: | kg | 0 | ✓ | Common | Text | Show Tier 2 - 18 | if(TAG_PARA_STATE_2_BOOL, PER_SUST_CARBON_FOOTPRINT_KG, "") |
| 19 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 19 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 20 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 20 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 21 | T3 | ARCH_TAG_7_PARA_WALL_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 21 | if(TAG_PARA_STATE_3_BOOL, ARCH_TAG_7_PARA_WALL_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | WARN_PER_THERM_U_VALUE_W_M2K_NR_WALLS | U > 0.70 W/m²K | Uganda NBC / ISO 6946 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, WARN_PER_THERM_U_VALUE_W_M2K_NR_WALLS, "") |
| 2 | CRITICAL | FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN | Fire resistance < 30 min | BS 9999 / Uganda NBC | Common | Text | Show Warning - 2 | if(TAG_WARN_VISIBLE_BOOL, FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN, "") |
| 3 | MEDIUM | WARN_PER_SUST_CARBON_FOOTPRINT_KG_WALLS__FLOORS__ | Embodied carbon > threshold | EDGE / IFC Performance | Common | Text | Show Warning - 3 | if(TAG_WARN_VISIBLE_BOOL, WARN_PER_SUST_CARBON_FOOTPRINT_KG_WALLS__FLOORS__, "") |

Family: STING - Floor Tag    TAG7 Container: ARCH_TAG_7_PARA_FLOOR_TXT

| Floors |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T1 | BLE_FLR_THICKNESS_MM | [ | mm] | 0 | ✓ | — | — | — | Add directly |
| 3 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 4 | T2 | BLE_FLR_TYPE_CLASSIFICATION_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, BLE_FLR_TYPE_CLASSIFICATION_TXT, "") |
| 5 | T2 | BLE_FLR_FINISH_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, BLE_FLR_FINISH_TXT, "") |
| 6 | T2 | BLE_FLR_LD_CAP_KPA |  |  | 0 |  | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, BLE_FLR_LD_CAP_KPA, "") |
| 7 | T2 | PER_THERM_U_VALUE_W_M2K_NR | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, PER_THERM_U_VALUE_W_M2K_NR, "") |
| 8 | T2 | FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN | FRR: |  | 0 |  | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN, "") |
| 9 | T2 | PER_SUST_RECYCLED_CONTENT_PCT | ¦ Recycled: | % | 0 |  | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, PER_SUST_RECYCLED_CONTENT_PCT, "") |
| 10 | T2 | PER_SUST_CARBON_FOOTPRINT_KG | ¦ CO₂: | kg | 0 | ✓ | Common | Text | Show Tier 2 - 10 | if(TAG_PARA_STATE_2_BOOL, PER_SUST_CARBON_FOOTPRINT_KG, "") |
| 11 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 11 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 12 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 12 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 13 | T3 | ARCH_TAG_7_PARA_FLOOR_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 13 | if(TAG_PARA_STATE_3_BOOL, ARCH_TAG_7_PARA_FLOOR_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | MEDIUM | WARN_PER_THERM_U_VALUE_W_M2K_NR_FLOORS | U > 0.70 W/m²K | Uganda NBC | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, WARN_PER_THERM_U_VALUE_W_M2K_NR_FLOORS, "") |
| 2 | CRITICAL | FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN | Fire resistance < 30 min | BS 9999 / Uganda NBC | Common | Text | Show Warning - 2 | if(TAG_WARN_VISIBLE_BOOL, FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN, "") |
| 3 | MEDIUM | WARN_PER_SUST_CARBON_FOOTPRINT_KG_WALLS__FLOORS__ | Embodied carbon > threshold | EDGE / IFC Performance | Common | Text | Show Warning - 3 | if(TAG_WARN_VISIBLE_BOOL, WARN_PER_SUST_CARBON_FOOTPRINT_KG_WALLS__FLOORS__, "") |

Family: STING - Ceiling Tag    TAG7 Container: ARCH_TAG_7_PARA_CEIL_TXT

| Ceilings |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T1 | BLE_CEILING_SYSTEM_TXT |  |  | 0 |  | — | — | — | Add directly |
| 3 | T1 | BLE_CEILING_HEIGHT_MM | [ | mm] | 1 | ✓ | — | — | — | Add directly |
| 4 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 5 | T2 | BLE_CEILING_SYSTEM_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, BLE_CEILING_SYSTEM_TXT, "") |
| 6 | T2 | BLE_CEILING_GRID_SZ_MM | ¦ Grid: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, BLE_CEILING_GRID_SZ_MM, "") |
| 7 | T2 | PER_THERM_U_VALUE_W_M2K_NR |  |  | 0 |  | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, PER_THERM_U_VALUE_W_M2K_NR, "") |
| 8 | T2 | FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN | ¦ FRR: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN, "") |
| 9 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 10 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 10 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 11 | T3 | ARCH_TAG_7_PARA_CEIL_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 11 | if(TAG_PARA_STATE_3_BOOL, ARCH_TAG_7_PARA_CEIL_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Roof Tag    TAG7 Container: ARCH_TAG_7_PARA_ROOF_TXT

| Roofs |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T1 | BLE_ROOF_COVERING_MAT_TXT |  |  | 0 |  | — | — | — | Add directly |
| 3 | T1 | BLE_ROOF_SLOPE_DEG |  |  | 1 | ✓ | — | — | — | Add directly |
| 4 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 5 | T2 | BLE_ROOF_TYPE_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, BLE_ROOF_TYPE_TXT, "") |
| 6 | T2 | BLE_ROOF_SLOPE_DEG | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, BLE_ROOF_SLOPE_DEG, "") |
| 7 | T2 | PER_THERM_U_VALUE_W_M2K_NR |  |  | 0 |  | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, PER_THERM_U_VALUE_W_M2K_NR, "") |
| 8 | T2 | FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN | ¦ FRR: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN, "") |
| 9 | T2 | PER_SUST_RECYCLED_CONTENT_PCT | Recycled: | % | 0 |  | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, PER_SUST_RECYCLED_CONTENT_PCT, "") |
| 10 | T2 | PER_SUST_CARBON_FOOTPRINT_KG | ¦ CO₂: | kg | 0 | ✓ | Common | Text | Show Tier 2 - 10 | if(TAG_PARA_STATE_2_BOOL, PER_SUST_CARBON_FOOTPRINT_KG, "") |
| 11 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 11 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 12 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 12 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 13 | T3 | ARCH_TAG_7_PARA_ROOF_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 13 | if(TAG_PARA_STATE_3_BOOL, ARCH_TAG_7_PARA_ROOF_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | WARN_PER_THERM_U_VALUE_W_M2K_NR_ROOFS | U > 0.50 W/m²K | Uganda NBC | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, WARN_PER_THERM_U_VALUE_W_M2K_NR_ROOFS, "") |
| 2 | CRITICAL | FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN | Fire resistance < 30 min | BS 9999 / Uganda NBC | Common | Text | Show Warning - 2 | if(TAG_WARN_VISIBLE_BOOL, FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN, "") |
| 3 | MEDIUM | WARN_PER_SUST_CARBON_FOOTPRINT_KG_WALLS__FLOORS__ | Embodied carbon > threshold | EDGE / IFC Performance | Common | Text | Show Warning - 3 | if(TAG_WARN_VISIBLE_BOOL, WARN_PER_SUST_CARBON_FOOTPRINT_KG_WALLS__FLOORS__, "") |

Family: STING - Door Tag    TAG7 Container: ARCH_TAG_7_PARA_DOOR_TXT

| Doors |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T1 | BLE_DOOR_WIDTH_MM |  |  | 0 |  | — | — | — | Add directly |
| 3 | T1 | BLE_DOOR_HEIGHT_MM |  |  | 0 | ✓ | — | — | — | Add directly |
| 4 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 5 | T2 | BLE_DOOR_OPERATION_TYPE_SWING_SLIDE_FOLD_ROLL_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, BLE_DOOR_OPERATION_TYPE_SWING_SLIDE_FOLD_ROLL_TXT, "") |
| 6 | T2 | BLE_DOOR_MAT_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, BLE_DOOR_MAT_TXT, "") |
| 7 | T2 | FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN | FRR: |  | 0 |  | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN, "") |
| 8 | T2 | BLE_DOOR_HARDWARE_SPECIFICATION_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, BLE_DOOR_HARDWARE_SPECIFICATION_TXT, "") |
| 9 | T2 | BLE_DOOR_FRAME_MAT_TXT | Frame: |  | 0 |  | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, BLE_DOOR_FRAME_MAT_TXT, "") |
| 10 | T2 | BLE_DOOR_GLAZING_TYPE_TXT | ¦ Glazing: |  | 0 | ✓ | Common | Text | Show Tier 2 - 10 | if(TAG_PARA_STATE_2_BOOL, BLE_DOOR_GLAZING_TYPE_TXT, "") |
| 11 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 11 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 12 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 12 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 13 | T3 | ARCH_TAG_7_PARA_DOOR_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 13 | if(TAG_PARA_STATE_3_BOOL, ARCH_TAG_7_PARA_DOOR_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | CRITICAL | BLE_DOOR_WIDTH_MM | Door width < 900 mm | Uganda PWD Act 2020 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, BLE_DOOR_WIDTH_MM, "") |
| 2 | CRITICAL | WARN_RGL_ACCESS_CLEAR_WIDTH_MM_DOORS__RAMPS__C | Clear width < 900 mm | Uganda PWD Act 2020 | Common | Text | Show Warning - 2 | if(TAG_WARN_VISIBLE_BOOL, WARN_RGL_ACCESS_CLEAR_WIDTH_MM_DOORS__RAMPS__C, "") |
| 3 | CRITICAL | FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN | Fire resistance < 30 min | BS 9999 / Uganda NBC | Common | Text | Show Warning - 3 | if(TAG_WARN_VISIBLE_BOOL, FLS_PROT_FLS_RESISTANCE_RATING_MINUTES_MIN, "") |

Family: STING - Window Tag    TAG7 Container: ARCH_TAG_7_PARA_WIN_TXT

| Windows |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T1 | BLE_WINDOW_WIDTH_MM |  |  | 0 |  | — | — | — | Add directly |
| 3 | T1 | BLE_WINDOW_HEIGHT_MM |  |  | 0 | ✓ | — | — | — | Add directly |
| 4 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 5 | T2 | BLE_WINDOW_FRAME_MAT_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, BLE_WINDOW_FRAME_MAT_TXT, "") |
| 6 | T2 | BLE_WINDOW_GLAZING_TYPE_SINGLE_DOUBLE_TRIPLE_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, BLE_WINDOW_GLAZING_TYPE_SINGLE_DOUBLE_TRIPLE_TXT, "") |
| 7 | T2 | BLE_WINDOW_U_VALUE_W_M_2K_NR |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, BLE_WINDOW_U_VALUE_W_M_2K_NR, "") |
| 8 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 9 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 9 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 10 | T3 | ARCH_TAG_7_PARA_WIN_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 10 | if(TAG_PARA_STATE_3_BOOL, ARCH_TAG_7_PARA_WIN_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | BLE_WINDOW_U_VALUE_W_M_2K_NR | U > 3.00 W/m²K | IEC / ASHRAE 90.1 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, BLE_WINDOW_U_VALUE_W_M_2K_NR, "") |
| 2 | HIGH | BLE_WINDOW_SOLAR_HEAT_GAIN_COEFFICIENT_NR | SHGC > 0.40 | ASHRAE 90.1 tropical | Common | Text | Show Warning - 2 | if(TAG_WARN_VISIBLE_BOOL, BLE_WINDOW_SOLAR_HEAT_GAIN_COEFFICIENT_NR, "") |

Family: STING - Stair Tag    TAG7 Container: ARCH_TAG_7_PARA_STAIR_TXT

| Stairs |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | BLE_STAIR_RISE_MM |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, BLE_STAIR_RISE_MM, "") |
| 4 | T2 | BLE_STAIR_GOING_MM | / |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, BLE_STAIR_GOING_MM, "") |
| 5 | T2 | BLE_STAIR_WIDTH_MM | ¦ Width: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, BLE_STAIR_WIDTH_MM, "") |
| 6 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 7 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 8 | T3 | ARCH_TAG_7_PARA_STAIR_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 8 | if(TAG_PARA_STATE_3_BOOL, ARCH_TAG_7_PARA_STAIR_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | BLE_STAIR_RISE_MM | Rise <150 OR >190 mm | BS 5395-1 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, BLE_STAIR_RISE_MM, "") |
| 2 | HIGH | BLE_STAIR_GOING_MM | Going <250 OR >400 mm | BS 5395-1 | Common | Text | Show Warning - 2 | if(TAG_WARN_VISIBLE_BOOL, BLE_STAIR_GOING_MM, "") |
| 3 | CRITICAL | BLE_STAIR_WIDTH_MM | Width < 1000 mm | BS 9999 / Uganda NBC | Common | Text | Show Warning - 3 | if(TAG_WARN_VISIBLE_BOOL, BLE_STAIR_WIDTH_MM, "") |

Family: STING - Ramp Tag    TAG7 Container: ARCH_TAG_7_PARA_RAMP_TXT

| Ramps |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | BLE_RAMP_SLOPE_PCT |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, BLE_RAMP_SLOPE_PCT, "") |
| 4 | T2 | BLE_RAMP_WIDTH_MM | ¦ Width: |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, BLE_RAMP_WIDTH_MM, "") |
| 5 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 6 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 7 | T3 | ARCH_TAG_7_PARA_RAMP_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, ARCH_TAG_7_PARA_RAMP_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | CRITICAL | WARN_BLE_RAMP_SLOPE_PCT_RAMPS | Slope > 8.33% (1:12) | Uganda PWD Act 2020 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, WARN_BLE_RAMP_SLOPE_PCT_RAMPS, "") |
| 2 | CRITICAL | WARN_RGL_ACCESS_CLEAR_WIDTH_MM_DOORS__RAMPS__C | Clear width < 900 mm | Uganda PWD Act 2020 | Common | Text | Show Warning - 2 | if(TAG_WARN_VISIBLE_BOOL, WARN_RGL_ACCESS_CLEAR_WIDTH_MM_DOORS__RAMPS__C, "") |

## 6B — Structure

Family: STING - Structural Column Tag    TAG7 Container: STR_TAG_7_PARA_COL_TXT

| Structural Columns |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | BLE_STRUCT_COLUMN_SZ_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_COLUMN_SZ_TXT, "") |
| 4 | T2 | BLE_STRUCT_MAT_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_MAT_TXT, "") |
| 5 | T2 | BLE_STRUCT_CONCRETE_GRADE_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_CONCRETE_GRADE_TXT, "") |
| 6 | T2 | BLE_STRUCT_LD_CAP_KN_NR | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_LD_CAP_KN_NR, "") |
| 7 | T2 | BLE_STRUCT_REINFORCEMENT_TXT | Rebar: |  | 0 | ✓ | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_REINFORCEMENT_TXT, "") |
| 8 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 9 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 9 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 10 | T3 | STR_TAG_7_PARA_COL_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 10 | if(TAG_PARA_STATE_3_BOOL, STR_TAG_7_PARA_COL_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | BLE_STRUCT_CONCRETE_GRADE_TXT | Grade vs exposure mismatch | BS 8500 / UNBS | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, BLE_STRUCT_CONCRETE_GRADE_TXT, "") |

Family: STING - Structural Framing Tag    TAG7 Container: STR_TAG_7_PARA_BEAM_TXT

| Structural Framing |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | BLE_STRUCT_BEAM_SZ_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_BEAM_SZ_TXT, "") |
| 4 | T2 | BLE_STRUCT_MAT_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_MAT_TXT, "") |
| 5 | T2 | BLE_STRUCT_CONCRETE_GRADE_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_CONCRETE_GRADE_TXT, "") |
| 6 | T2 | BLE_STRUCT_REINFORCEMENT_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_REINFORCEMENT_TXT, "") |
| 7 | T2 | BLE_STRUCT_LD_CAP_KN_NR | Capacity: |  | 0 | ✓ | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_LD_CAP_KN_NR, "") |
| 8 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 9 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 9 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 10 | T3 | STR_TAG_7_PARA_BEAM_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 10 | if(TAG_PARA_STATE_3_BOOL, STR_TAG_7_PARA_BEAM_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | BLE_STRUCT_CONCRETE_GRADE_TXT | Grade vs exposure mismatch | BS 8500 / UNBS | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, BLE_STRUCT_CONCRETE_GRADE_TXT, "") |

Family: STING - Structural Foundation Tag    TAG7 Container: STR_TAG_7_PARA_FDN_TXT

| Structural Foundations |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | BLE_STRUCT_FOUNDATION_TYPE_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_FOUNDATION_TYPE_TXT, "") |
| 4 | T2 | BLE_STRUCT_FOUNDATION_SZ_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_FOUNDATION_SZ_TXT, "") |
| 5 | T2 | BLE_STRUCT_FOUNDATION_DEPTH_MM | Depth: | mm | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_FOUNDATION_DEPTH_MM, "") |
| 6 | T2 | BLE_STRUCT_CONCRETE_GRADE_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_CONCRETE_GRADE_TXT, "") |
| 7 | T2 | BLE_STRUCT_LD_CAP_KN_NR |  |  | 0 |  | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_LD_CAP_KN_NR, "") |
| 8 | T2 | BLE_STRUCT_REINFORCEMENT_TXT | ¦ Rebar: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, BLE_STRUCT_REINFORCEMENT_TXT, "") |
| 9 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 10 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 10 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 11 | T3 | STR_TAG_7_PARA_FDN_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 11 | if(TAG_PARA_STATE_3_BOOL, STR_TAG_7_PARA_FDN_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | BLE_STRUCT_CONCRETE_GRADE_TXT | Grade vs exposure mismatch | BS 8500 / UNBS | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, BLE_STRUCT_CONCRETE_GRADE_TXT, "") |

## 6C — Mechanical / HVAC

Family: STING - Mechanical Equipment Tag    TAG7 Container: HVC_TAG_7_PARA_SPEC_TXT

| Mechanical Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ASS_DESCRIPTION_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 4 | T2 | HVC_CAP_KW |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, HVC_CAP_KW, "") |
| 5 | T2 | HVC_PWR_KW | / |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, HVC_PWR_KW, "") |
| 6 | T2 | HVC_DCT_FLW_CFM | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, HVC_DCT_FLW_CFM, "") |
| 7 | T2 | HVC_REFRIGERANT_TXT | Ref: |  | 0 |  | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, HVC_REFRIGERANT_TXT, "") |
| 8 | T2 | HVC_EFF_RATIO_NR | ¦ EER: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, HVC_EFF_RATIO_NR, "") |
| 9 | T2 | HVC_CONTROL_TYPE_TXT | BMS: |  | 0 |  | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, HVC_CONTROL_TYPE_TXT, "") |
| 10 | T2 | MNT_HGT_MM | ¦ Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 10 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 11 | T2 | ASS_MANUFACTURER_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 11 | if(TAG_PARA_STATE_2_BOOL, ASS_MANUFACTURER_TXT, "") |
| 12 | T2 | ASS_MODEL_NR_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 12 | if(TAG_PARA_STATE_2_BOOL, ASS_MODEL_NR_TXT, "") |
| 13 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 13 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 14 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 14 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 15 | T3 | HVC_TAG_7_PARA_SPEC_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 15 | if(TAG_PARA_STATE_3_BOOL, HVC_TAG_7_PARA_SPEC_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | WARN_HVC_EFF_RATIO_NR_MECHANICAL_EQUI | COP < 3.0 | ASHRAE 90.1 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, WARN_HVC_EFF_RATIO_NR_MECHANICAL_EQUI, "") |

Family: STING - Duct Tag    TAG7 Container: HVC_TAG_7_PARA_DUCT_TXT

| Ducts |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T1 | HVC_DCT_PROPERTY_TXT | [ | ] | 0 | ✓ | — | — | — | Add directly |
| 3 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 4 | T2 | HVC_DCT_FLW_CFM |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, HVC_DCT_FLW_CFM, "") |
| 5 | T2 | HVC_VEL_MPS | ¦ |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, HVC_VEL_MPS, "") |
| 6 | T2 | HVC_PIPE_PRESSURE_KPA | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, HVC_PIPE_PRESSURE_KPA, "") |
| 7 | T2 | HVC_DCT_MAT_TXT | Mat: |  | 0 |  | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, HVC_DCT_MAT_TXT, "") |
| 8 | T2 | SYS_INSULATION_TYPE_TXT | ¦ Insul: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, SYS_INSULATION_TYPE_TXT, "") |
| 9 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 10 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 10 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 11 | T3 | HVC_TAG_7_PARA_DUCT_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 11 | if(TAG_PARA_STATE_3_BOOL, HVC_TAG_7_PARA_DUCT_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | HVC_VEL_MPS | Air velocity > 6.0 m/s | CIBSE Guide B / ASHRAE | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, HVC_VEL_MPS, "") |

Family: STING - Duct Fitting Tag    TAG7 Container: HVC_TAG_7_PARA_DCTACC_TXT

| Duct Fittings |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | HVC_DCT_PROPERTY_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, HVC_DCT_PROPERTY_TXT, "") |
| 4 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 5 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 5 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 6 | T3 | HVC_TAG_7_PARA_DCTACC_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, HVC_TAG_7_PARA_DCTACC_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Duct Accessory Tag    TAG7 Container: HVC_TAG_7_PARA_DCTACC_TXT

| Duct Accessories |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ASS_DESCRIPTION_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 4 | T2 | HVC_DCT_PROPERTY_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, HVC_DCT_PROPERTY_TXT, "") |
| 5 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 6 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 7 | T3 | HVC_TAG_7_PARA_DCTACC_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, HVC_TAG_7_PARA_DCTACC_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Air Terminal Tag    TAG7 Container: HVC_TAG_7_PARA_AT_TXT

| Air Terminals |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ASS_DESCRIPTION_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 4 | T2 | HVC_DCT_FLW_CFM |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, HVC_DCT_FLW_CFM, "") |
| 5 | T2 | HVC_VEL_MPS | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, HVC_VEL_MPS, "") |
| 6 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 7 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 8 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 8 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 9 | T3 | HVC_TAG_7_PARA_AT_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 9 | if(TAG_PARA_STATE_3_BOOL, HVC_TAG_7_PARA_AT_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | MEDIUM | HVC_DCT_SOUNDLVL_DB | Terminal noise > 35 dB | CIBSE Guide A | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, HVC_DCT_SOUNDLVL_DB, "") |

Family: STING - Flex Duct Tag    TAG7 Container: HVC_TAG_7_PARA_FLEXDUCT_TXT

| Flex Ducts |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | HVC_DCT_FLW_CFM |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, HVC_DCT_FLW_CFM, "") |
| 4 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 5 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 5 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 6 | T3 | HVC_TAG_7_PARA_FLEXDUCT_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, HVC_TAG_7_PARA_FLEXDUCT_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | WARN_HVC_VEL_MPS_FLEX_DUCTS | Flex duct vel > 4.0 m/s | SMACNA | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, WARN_HVC_VEL_MPS_FLEX_DUCTS, "") |

## 6D — Electrical

Family: STING - Electrical Equipment Tag    TAG7 Container: ELC_TAG_7_PARA_PANEL_TXT

| Electrical Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T1 | ELC_PNL_DESIGNATION_NAME_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 3 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 4 | T2 | ELC_PNL_VLT_V |  | V | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ELC_PNL_VLT_V, "") |
| 5 | T2 | ELC_PNL_PHS_COUNT_NR | ¦ | Ph | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ELC_PNL_PHS_COUNT_NR, "") |
| 6 | T2 | ELC_PNL_NUM_OF_WAYS_NR | ¦ | Ways | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, ELC_PNL_NUM_OF_WAYS_NR, "") |
| 7 | T2 | ELC_PNL_RATED_KW |  |  | 0 |  | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, ELC_PNL_RATED_KW, "") |
| 8 | T2 | ELC_PNL_CONNECTED_LOAD_KW | ¦ Load: | kW | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, ELC_PNL_CONNECTED_LOAD_KW, "") |
| 9 | T2 | ELC_PNL_MAIN_BRK_A | MCB: | A | 0 |  | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, ELC_PNL_MAIN_BRK_A, "") |
| 10 | T2 | ELC_SPARE_WAYS_NR | ¦ Spare: |  | 0 | ✓ | Common | Text | Show Tier 2 - 10 | if(TAG_PARA_STATE_2_BOOL, ELC_SPARE_WAYS_NR, "") |
| 11 | T2 | ELC_VLT_DROP_PCT | VD: | % | 0 |  | Common | Text | Show Tier 2 - 11 | if(TAG_PARA_STATE_2_BOOL, ELC_VLT_DROP_PCT, "") |
| 12 | T2 | ELC_PNL_SHORT_CIRCUIT_RATING_KA | ¦ Fault: | kA | 0 | ✓ | Common | Text | Show Tier 2 - 12 | if(TAG_PARA_STATE_2_BOOL, ELC_PNL_SHORT_CIRCUIT_RATING_KA, "") |
| 13 | T2 | ELC_CBL_FEEDER_SZ_MM2 | Feeder: | mm² | 0 |  | Common | Text | Show Tier 2 - 13 | if(TAG_PARA_STATE_2_BOOL, ELC_CBL_FEEDER_SZ_MM2, "") |
| 14 | T2 | ELC_IP_RATING_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 14 | if(TAG_PARA_STATE_2_BOOL, ELC_IP_RATING_TXT, "") |
| 15 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 15 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 16 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 16 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 17 | T3 | ELC_TAG_7_PARA_PANEL_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 17 | if(TAG_PARA_STATE_3_BOOL, ELC_TAG_7_PARA_PANEL_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | CRITICAL | WARN_ELC_VLT_DROP_PCT_ELECTRICAL_EQUI | Voltage drop > 4.0% | IEC 60364-5-52 / BS 7671 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, WARN_ELC_VLT_DROP_PCT_ELECTRICAL_EQUI, "") |
| 2 | CRITICAL | ELC_PNL_CONNECTED_LOAD_KW | Load > 80% of rated kW | Design practice | Common | Text | Show Warning - 2 | if(TAG_WARN_VISIBLE_BOOL, ELC_PNL_CONNECTED_LOAD_KW, "") |
| 3 | MEDIUM | ELC_PNL_SHORT_CIRCUIT_RATING_KA | Fault level < 6 kA | IEC 60947-2 | Common | Text | Show Warning - 3 | if(TAG_WARN_VISIBLE_BOOL, ELC_PNL_SHORT_CIRCUIT_RATING_KA, "") |

Family: STING - Electrical Fixture Tag    TAG7 Container: ELC_TAG_7_PARA_CIRCUIT_TXT

| Electrical Fixtures |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ELC_PWR_KW |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ELC_PWR_KW, "") |
| 4 | T2 | ELC_VLT_PRIMARY_RATING_V | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ELC_VLT_PRIMARY_RATING_V, "") |
| 5 | T2 | ELC_CKT_NR |  |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ELC_CKT_NR, "") |
| 6 | T2 | ELC_PNL_DESIGNATION_NAME_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, ELC_PNL_DESIGNATION_NAME_TXT, "") |
| 7 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 8 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 9 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 9 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 10 | T3 | ELC_TAG_7_PARA_CIRCUIT_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 10 | if(TAG_PARA_STATE_3_BOOL, ELC_TAG_7_PARA_CIRCUIT_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | CRITICAL | WARN_ELC_VLT_DROP_PCT_ELECTRICAL_EQUI | Voltage drop > 4.0% | IEC 60364-5-52 / BS 7671 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, WARN_ELC_VLT_DROP_PCT_ELECTRICAL_EQUI, "") |

Family: STING - Lighting Fixture Tag    TAG7 Container: ELC_TAG_7_PARA_CIRCUIT_TXT

| Lighting Fixtures |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T1 | LTG_FIX_TYPE_CLASSIFICATION_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 3 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 4 | T2 | LTG_FIX_LMP_WATTAGE_W |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, LTG_FIX_LMP_WATTAGE_W, "") |
| 5 | T2 | CST_FIX_LUMEN_OUTPUT_LM | ¦ |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, CST_FIX_LUMEN_OUTPUT_LM, "") |
| 6 | T2 | LTG_CLR_TEMP_K | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, LTG_CLR_TEMP_K, "") |
| 7 | T2 | LTG_EFFICACY_LM_W | Eff: |  | 0 |  | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, LTG_EFFICACY_LM_W, "") |
| 8 | T2 | ELC_CKT_NR | ¦ Ckt: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, ELC_CKT_NR, "") |
| 9 | T2 | ASS_MANUFACTURER_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, ASS_MANUFACTURER_TXT, "") |
| 10 | T2 | ASS_MODEL_NR_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 10 | if(TAG_PARA_STATE_2_BOOL, ASS_MODEL_NR_TXT, "") |
| 11 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 11 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 12 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 12 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 13 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 13 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 14 | T3 | ELC_TAG_7_PARA_CIRCUIT_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 14 | if(TAG_PARA_STATE_3_BOOL, ELC_TAG_7_PARA_CIRCUIT_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | MEDIUM | ASS_DESIGN_LTG_LVL_LUX_NR | Illuminance < 300 lux | BS EN 12464-1 / CIBSE | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, ASS_DESIGN_LTG_LVL_LUX_NR, "") |

Family: STING - Lighting Device Tag    TAG7 Container: ELC_TAG_7_PARA_CIRCUIT_TXT

| Lighting Devices |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ELC_CKT_NR |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ELC_CKT_NR, "") |
| 4 | T2 | ELC_PNL_DESIGNATION_NAME_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ELC_PNL_DESIGNATION_NAME_TXT, "") |
| 5 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 6 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 7 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 8 | T3 | ELC_TAG_7_PARA_CIRCUIT_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 8 | if(TAG_PARA_STATE_3_BOOL, ELC_TAG_7_PARA_CIRCUIT_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Conduit Tag    TAG7 Container: ELC_TAG_7_PARA_CONDUIT_TXT

| Conduits |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T1 | ELC_CDT_SZ_MM |  |  | 0 | ✓ | — | — | — | Add directly |
| 3 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 4 | T2 | ELC_CKT_NR |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ELC_CKT_NR, "") |
| 5 | T2 | ELC_CDT_MAT_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ELC_CDT_MAT_TXT, "") |
| 6 | T2 | ELC_CBL_SOURCE_FROM_TXT | From: |  | 0 |  | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, ELC_CBL_SOURCE_FROM_TXT, "") |
| 7 | T2 | ELC_CBL_DESTINATION_TO_TXT | → |  | 0 | ✓ | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, ELC_CBL_DESTINATION_TO_TXT, "") |
| 8 | T2 | ELC_CBL_INSTALL_METHOD_TXT | Install: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, ELC_CBL_INSTALL_METHOD_TXT, "") |
| 9 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 10 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 10 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 11 | T3 | ELC_TAG_7_PARA_CONDUIT_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 11 | if(TAG_PARA_STATE_3_BOOL, ELC_TAG_7_PARA_CONDUIT_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | ELC_CDT_CBL_FILL_PCT | Conduit fill > 40% | IEC 60364 / BS 7671 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, ELC_CDT_CBL_FILL_PCT, "") |

Family: STING - Conduit Fitting Tag    TAG7 Container: ELC_TAG_7_PARA_CONDUIT_TXT

| Conduit Fittings |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ELC_CDT_SZ_MM |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ELC_CDT_SZ_MM, "") |
| 4 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 5 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 5 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 6 | T3 | ELC_TAG_7_PARA_CONDUIT_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, ELC_TAG_7_PARA_CONDUIT_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Cable Tray Tag    TAG7 Container: ELC_TAG_7_PARA_TRAY_TXT

| Cable Trays |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ELC_CTR_MAT_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ELC_CTR_MAT_TXT, "") |
| 4 | T2 | MNT_HGT_MM | ¦ Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 5 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 6 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 7 | T3 | ELC_TAG_7_PARA_TRAY_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, ELC_TAG_7_PARA_TRAY_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | ELC_CTR_FILL_PCT | Cable tray fill > 45% | IEC 61537 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, ELC_CTR_FILL_PCT, "") |

Family: STING - Cable Tray Fitting Tag    TAG7 Container: ELC_TAG_7_PARA_TRAY_TXT

| Cable Tray Fittings |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ELC_CTR_MAT_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ELC_CTR_MAT_TXT, "") |
| 4 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 5 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 5 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 6 | T3 | ELC_TAG_7_PARA_TRAY_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, ELC_TAG_7_PARA_TRAY_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

## 6E — Plumbing

Family: STING - Plumbing Fixture Tag    TAG7 Container: PLM_TAG_7_PARA_FIXTURE_TXT

| Plumbing Fixtures |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | PLM_PPE_SZ_MM |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, PLM_PPE_SZ_MM, "") |
| 4 | T2 | PLM_PPE_FLW_LPS | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, PLM_PPE_FLW_LPS, "") |
| 5 | T2 | ASS_MANUFACTURER_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ASS_MANUFACTURER_TXT, "") |
| 6 | T2 | ASS_MODEL_NR_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, ASS_MODEL_NR_TXT, "") |
| 7 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 8 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 8 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 9 | T3 | PLM_TAG_7_PARA_FIXTURE_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 9 | if(TAG_PARA_STATE_3_BOOL, PLM_TAG_7_PARA_FIXTURE_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | MEDIUM | PLM_PSR_KPA | Pressure <150 OR >600 kPa | NWSC supply | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, PLM_PSR_KPA, "") |

Family: STING - Pipe Tag    TAG7 Container: PLM_TAG_7_PARA_PIPE_TXT

| Pipes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T1 | PLM_PPE_SZ_MM |  |  | 0 | ✓ | — | — | — | Add directly |
| 3 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 4 | T2 | PLM_PPE_FLW_LPS |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, PLM_PPE_FLW_LPS, "") |
| 5 | T2 | PLM_VEL_MPS | ¦ |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, PLM_VEL_MPS, "") |
| 6 | T2 | PLM_PSR_KPA | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, PLM_PSR_KPA, "") |
| 7 | T2 | PLM_PPE_MAT_TXT | Mat: |  | 0 |  | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, PLM_PPE_MAT_TXT, "") |
| 8 | T2 | SYS_INSULATION_TYPE_TXT | ¦ Insul: |  | 0 | ✓ | Common | Text | Show Tier 2 - 8 | if(TAG_PARA_STATE_2_BOOL, SYS_INSULATION_TYPE_TXT, "") |
| 9 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 9 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 10 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 10 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 11 | T3 | PLM_TAG_7_PARA_PIPE_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 11 | if(TAG_PARA_STATE_3_BOOL, PLM_TAG_7_PARA_PIPE_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | PLM_VEL_MPS | Pipe velocity > 2.0 m/s | BS EN 806-3 / CIBSE | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, PLM_VEL_MPS, "") |
| 2 | MEDIUM | WARN_PLM_PPE_FLW_LPS_PIPES__HOT_WATE | Hot water flow > 2.0 L/s | BS EN 806-3 | Common | Text | Show Warning - 2 | if(TAG_WARN_VISIBLE_BOOL, WARN_PLM_PPE_FLW_LPS_PIPES__HOT_WATE, "") |
| 3 | MEDIUM | PLM_PSR_KPA | Pressure <150 OR >600 kPa | NWSC supply | Common | Text | Show Warning - 3 | if(TAG_WARN_VISIBLE_BOOL, PLM_PSR_KPA, "") |

Family: STING - Pipe Fitting Tag    TAG7 Container: PLM_TAG_7_PARA_PIPEACC_TXT

| Pipe Fittings |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | PLM_PPE_SZ_MM |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, PLM_PPE_SZ_MM, "") |
| 4 | T2 | PLM_PPE_MAT_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, PLM_PPE_MAT_TXT, "") |
| 5 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 6 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 7 | T3 | PLM_TAG_7_PARA_PIPEACC_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, PLM_TAG_7_PARA_PIPEACC_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Pipe Accessory Tag    TAG7 Container: PLM_TAG_7_PARA_PIPEACC_TXT

| Pipe Accessories |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | PLM_PPE_SZ_MM |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, PLM_PPE_SZ_MM, "") |
| 4 | T2 | ASS_DESCRIPTION_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 5 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 6 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 7 | T3 | PLM_TAG_7_PARA_PIPEACC_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, PLM_TAG_7_PARA_PIPEACC_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Flex Pipe Tag    TAG7 Container: PLM_TAG_7_PARA_PIPE_TXT

| Flex Pipes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | PLM_PPE_SZ_MM |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, PLM_PPE_SZ_MM, "") |
| 4 | T2 | PLM_PPE_MAT_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, PLM_PPE_MAT_TXT, "") |
| 5 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 6 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 7 | T3 | PLM_TAG_7_PARA_PIPE_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, PLM_TAG_7_PARA_PIPE_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

## 6F — Fire Protection

Family: STING - Sprinkler Tag    TAG7 Container: FLS_TAG_7_PARA_SPR_TXT

| Sprinklers |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | FLS_SFTY_COVERAGE_AREA_SQ_M |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, FLS_SFTY_COVERAGE_AREA_SQ_M, "") |
| 4 | T2 | FLS_SFTY_K_FACTOR_NR | ¦ K= |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, FLS_SFTY_K_FACTOR_NR, "") |
| 5 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 6 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 7 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 8 | T3 | FLS_TAG_7_PARA_SPR_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 8 | if(TAG_PARA_STATE_3_BOOL, FLS_TAG_7_PARA_SPR_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | FLS_SFTY_TEMP_RATING_C | Temp <57°C OR >79°C | BS EN 12845 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, FLS_SFTY_TEMP_RATING_C, "") |
| 2 | HIGH | WARN_FLS_SFTY_COVERAGE_AREA_SQ_M_SPRINKLERS__FIR | Coverage > 21 m² | BS EN 12845 / BS 5839 | Common | Text | Show Warning - 2 | if(TAG_WARN_VISIBLE_BOOL, WARN_FLS_SFTY_COVERAGE_AREA_SQ_M_SPRINKLERS__FIR, "") |

Family: STING - Fire Alarm Device Tag    TAG7 Container: FLS_TAG_7_PARA_FA_TXT

| Fire Alarm Devices |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | FLS_SFTY_DEV_TYPE_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, FLS_SFTY_DEV_TYPE_TXT, "") |
| 4 | T2 | FLS_SFTY_DEV_ZONE_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, FLS_SFTY_DEV_ZONE_TXT, "") |
| 5 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 6 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 7 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 8 | T3 | FLS_TAG_7_PARA_FA_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 8 | if(TAG_PARA_STATE_3_BOOL, FLS_TAG_7_PARA_FA_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | HIGH | WARN_FLS_SFTY_COVERAGE_AREA_SQ_M_SPRINKLERS__FIR | Coverage > 21 m² | BS EN 12845 / BS 5839 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, WARN_FLS_SFTY_COVERAGE_AREA_SQ_M_SPRINKLERS__FIR, "") |

## 6G — Specialist / LV

Family: STING - Communication Device Tag    TAG7 Container: COM_TAG_7_PARA_BMS_TXT

| Communication Devices |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | COM_C_PROTOCOL_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, COM_C_PROTOCOL_TXT, "") |
| 4 | T2 | COM_C_NETWORK_ADDRESS_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, COM_C_NETWORK_ADDRESS_TXT, "") |
| 5 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 6 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 7 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 8 | T3 | COM_TAG_7_PARA_BMS_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 8 | if(TAG_PARA_STATE_3_BOOL, COM_TAG_7_PARA_BMS_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Data Device Tag    TAG7 Container: ICT_TAG_7_PARA_DATA_TXT

| Data Devices |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ICT_OUTLET_TYPE_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ICT_OUTLET_TYPE_TXT, "") |
| 4 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 5 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 6 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 7 | T3 | ICT_TAG_7_PARA_DATA_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, ICT_TAG_7_PARA_DATA_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Nurse Call Device Tag    TAG7 Container: NCL_TAG_7_PARA_NURSECALL_TXT

| Nurse Call Devices |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | NCL_ZONE_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, NCL_ZONE_TXT, "") |
| 4 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 5 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 6 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 7 | T3 | NCL_TAG_7_PARA_NURSECALL_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, NCL_TAG_7_PARA_NURSECALL_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Security Device Tag    TAG7 Container: SEC_TAG_7_PARA_SECURITY_TXT

| Security Devices |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | SEC_DEV_ZONE_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, SEC_DEV_ZONE_TXT, "") |
| 4 | T2 | ASS_DESCRIPTION_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 5 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 6 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 7 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 8 | T3 | SEC_TAG_7_PARA_SECURITY_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 8 | if(TAG_PARA_STATE_3_BOOL, SEC_TAG_7_PARA_SECURITY_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Telephone Device Tag    TAG7 Container: ICT_TAG_7_PARA_DATA_TXT

| Telephone Devices |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ICT_OUTLET_TYPE_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ICT_OUTLET_TYPE_TXT, "") |
| 4 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 5 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 6 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 6 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 7 | T3 | ICT_TAG_7_PARA_DATA_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 7 | if(TAG_PARA_STATE_3_BOOL, ICT_TAG_7_PARA_DATA_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

## 6H — Generic & Spaces

Family: STING - Room Tag

| Rooms |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ASS_DESCRIPTION_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 4 | T2 | ASS_LVL_COD_TXT | Level: |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_LVL_COD_TXT, "") |
| 5 | T2 | ASS_LOC_TXT | ¦ Loc: |  | 0 |  | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ASS_LOC_TXT, "") |
| 6 | T2 | ASS_ZONE_TXT | ¦ Zone: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, ASS_ZONE_TXT, "") |
| 7 | T2 | ASS_ROOM_AREA_SQ_M | Area: | m² | 0 | ✓ | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, ASS_ROOM_AREA_SQ_M, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| 1 | MEDIUM | PER_ACOUSTICS_BACKGROUND_NOISE_DB | Background noise > 40 dB | CIBSE Guide A / BB93 | Common | Text | Show Warning - 1 | if(TAG_WARN_VISIBLE_BOOL, PER_ACOUSTICS_BACKGROUND_NOISE_DB, "") |
| 2 | MEDIUM | ASS_DESIGN_LTG_LVL_LUX_NR | Illuminance < 300 lux | BS EN 12464-1 / CIBSE | Common | Text | Show Warning - 2 | if(TAG_WARN_VISIBLE_BOOL, ASS_DESIGN_LTG_LVL_LUX_NR, "") |

Family: STING - Furniture Tag

| Furniture |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ASS_DESCRIPTION_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 4 | T2 | ASS_MANUFACTURER_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_MANUFACTURER_TXT, "") |
| 5 | T2 | ASS_MODEL_NR_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ASS_MODEL_NR_TXT, "") |
| 6 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Furniture System Tag

| Furniture Systems |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ASS_DESCRIPTION_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 4 | T2 | ASS_MANUFACTURER_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_MANUFACTURER_TXT, "") |
| 5 | T2 | ASS_MODEL_NR_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ASS_MODEL_NR_TXT, "") |
| 6 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Casework Tag

| Casework |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ASS_DESCRIPTION_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 4 | T2 | ASS_MANUFACTURER_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_MANUFACTURER_TXT, "") |
| 5 | T2 | ASS_MODEL_NR_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ASS_MODEL_NR_TXT, "") |
| 6 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Generic Model Tag    TAG7 Container: ASS_TAG_7_PARA_EQUIPMENT_TXT

| Generic Models |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ASS_DESCRIPTION_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 4 | T2 | ASS_MANUFACTURER_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_MANUFACTURER_TXT, "") |
| 5 | T2 | ASS_MODEL_NR_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ASS_MODEL_NR_TXT, "") |
| 6 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 7 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 8 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 8 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 9 | T3 | ASS_TAG_7_PARA_EQUIPMENT_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 9 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_7_PARA_EQUIPMENT_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Specialty Equipment Tag    TAG7 Container: ASS_TAG_7_PARA_EQUIPMENT_TXT

| Specialty Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ASS_DESCRIPTION_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 4 | T2 | ASS_MANUFACTURER_TXT |  |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_MANUFACTURER_TXT, "") |
| 5 | T2 | ASS_MODEL_NR_TXT | ¦ |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ASS_MODEL_NR_TXT, "") |
| 6 | T2 | MNT_HGT_MM | Ht: | mm | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, MNT_HGT_MM, "") |
| 7 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 7 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| 8 | T3 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 8 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_2_TXT, "") |
| 9 | T3 | ASS_TAG_7_PARA_EQUIPMENT_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 3 - 9 | if(TAG_PARA_STATE_3_BOOL, ASS_TAG_7_PARA_EQUIPMENT_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Parking Tag

| Parking |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ASS_DESCRIPTION_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 4 | T2 | ASS_LVL_COD_TXT | Lvl: |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_LVL_COD_TXT, "") |
| 5 | T2 | ASS_ZONE_TXT | ¦ Zone: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ASS_ZONE_TXT, "") |
| 6 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |

Family: STING - Site Tag

| Site |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # | Tier | Parameter Name | Prefix | Suffix | Spc | Brk | Discipline | Type | Name (in Calculated Value dialog) | Calculated Value Formula (type exactly) |
| 1 | T1 | ASS_TAG_1_TXT |  |  | 0 | ✓ | — | — | — | Add directly |
| 2 | T2 | ASS_TAG_2_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 2 | if(TAG_PARA_STATE_2_BOOL, ASS_TAG_2_TXT, "") |
| 3 | T2 | ASS_DESCRIPTION_TXT |  |  | 0 | ✓ | Common | Text | Show Tier 2 - 3 | if(TAG_PARA_STATE_2_BOOL, ASS_DESCRIPTION_TXT, "") |
| 4 | T2 | ASS_LOC_TXT | Loc: |  | 0 |  | Common | Text | Show Tier 2 - 4 | if(TAG_PARA_STATE_2_BOOL, ASS_LOC_TXT, "") |
| 5 | T2 | ASS_ZONE_TXT | ¦ Zone: |  | 0 | ✓ | Common | Text | Show Tier 2 - 5 | if(TAG_PARA_STATE_2_BOOL, ASS_ZONE_TXT, "") |
| 6 | T2 | RGL_STD_TXT | Std: |  | 0 | ✓ | Common | Text | Show Tier 2 - 6 | if(TAG_PARA_STATE_2_BOOL, RGL_STD_TXT, "") |
| ⚠ WARNING PARAMETERS — add after all Tier rows (Break = ✓, Calculated Value gates visibility) |  |  |  |  |  |  |
| # | Sev | Warning Parameter Name | Threshold Condition | Standard | Dis. | Type | Name | Calculated Value Formula |
| No warning parameters for this category. |  |  |  |  |  |  |